# DSR-C100		Architect & Engineer Specification___________________

## Bid Spec

- Description

The color digital recording camera shall be a Sanyo Model number DSR-C100 or equivalent. The camera described shall be an industrial grade, high sensitivity, high-resolution, with a built-in hard disk drive.

- Bid Specification

The color camera shall incorporate a ½” 1.5 million pixel progressive scan, charge coupled device (CCD). The color digital recording camera shall produce a digital image resolution of 1360 (H) X 1024 (V) pixels (SXGA). The camera shall have a minimum light requirement of approximately 15 lux with a built in lens.

The built-in lens shall have a fixed focus, focal range of 2.3m and focal length of  f = 7.0mm (equivalent of  f = 38mm lens on 35mm camera) The camera shall have automatic white balance, exposure, and gain control (ISO level 100~800). The video output level shall be 1.0 Vp-p (75 ohms composite) NTSC / PAL selectable with a BNC type connection.

The camera shall have a built-in 3.5” 10.2 GB HDD (hard disk drive) (Proprietary format) with a JPEG image file format (Exif Ver. 2.1 compliant) The alarm mode recording capacity shall be 10-90 minutes (variable and allocatable to pre/post) The camera shall have pre-alarm and post-alarm recording capability of 5-85 minutes. The camera shall have a suspicion mode capable of 502-11,565 frames (variable according to alarm recording time and suspicion recording rate).  The camera shall have a built-in archive capable of storing 500 frames.

The camera shall have a digital zoom from 1x to 21  x (7 steps). The camera shall have a normal playback at 3 fps (frames per second) with optional selection to 1 fps (frame per second).

The camera shall have a USB type B connector, an RS-232C (D-Sub 9-pin) connector, and a video (BNC) connector.

The camera shall not exceed a dimension of 4.8”W x 4.61”H x 9.33”D and weigh no more than 89.9 0z. (2,550 grams). (w/o camera mount) Power requirements for the video camera shall be 12V ~ 16V DC, 0.9A ~ 0.7A and shall consume approx. 10.5W. The video camera shall be constructed of a durable metal cabinet and shall be magnetically and electrostatically shielded. The camera shall feature solid state components to resist shock and vibration. The video camera shall be ISO9001 certified and UL listed.

- Minimum Performance Specifications

| Specifications |  |
| --- | --- |
| Model | DSR-C100 |
| Image Sensor | 1/2" 1.5 million pixel CCD, progressive scan |
| Picture Elements | Total: 1434(H) x 1050(V), Effective: 1360(H) x 1024(V) |
| Shooting Number | Alarm Mode: 3fps (for both pre-alarm / post-alarm) |
|  | Suspicion mode: 1 or 3 fps (selectable) |
| Minimum Illumination | Approximately 15 lux |
| Backlight Compensation | ON: Multi-spot photometry (64-section) / OFF - (on screen) |
| Flicker less | ON / OFF - (on screen) |
| White Balance | Automatic |
| Exposure | Automatic |
| Gain Control | Automatic (ISO level 100 ~ 800) |
| Electronic Shutter speed | Automatic (1/50 ~ 1/10,000), (1/50 ~ 1/100) |
| Structure | Five elements in five groups |
| Built-In Lens Focus | Fixed focus |
| Focal Length | f = 7.0 mm (equivalent to f = 38mm lens on 35 mm camera) |
| Focal Range | 2.3m ~ infinite |
| Recording Media | 3.5" 10.2 GB HDD (Proprietary format) |
| Resolution of digital image | SXGA: 1360(H) x 1024(V) pixels |
| Image file format | JPEG compliant (Exif Ver. 2.1 compliant) |
|  | Alarm recording time: 10 ~ 90 min. (variable, and allocatable to pre/post |
| Alarm Mode | Pre-alarm recording time: 5 ~ 85 min. |
| Recording | Post-alarm recording time: 5 ~ 85 min. |
| Capacity Suspicion Mode | 502 ~ 11,565 frames |
|  | (variable according to alarm recording time and suspicion recording rate) |
| Archive Mode | Max. 500 frames |
|  | Alarm playback: 4/9 screen |
| Playback Split Screen | Suspicion playback: 6/9 screen |
| Functions | Archive / Extension playback: 6 screen |
| Normal Playback | 3 fps |
| Searching | Skip search (forward and reverse a 5/15/30 sec. Interval) |
| Digital Zoom | 1x to 21 x (7 steps) |
| Hold-Up input | Low level active (0v, 1 sec, or more) |
| Suspicion input | Low level active (0v, 100 msec, or more) |
| Video output | VBS 1.0V (p-p) (75ohms, composite) NTSC / PAL selectable |
| Warning output | High level active (DC 3v±0.3v) |
| USB | Type B connector (rear) |
| Interface RS-232C | D-sub 9-pin (rear) |
| Video Signal | BNC (rear) |
| Power Supply | 2-pin terminal (rear) |
| Extension Slot | CF type II (side), for a CompactFlash or a Microdrive (3,3v) |
| Operating Conditions | Temperature: 5 degrees C to 40 degrees C |
|  | Humidity: 10% to 80%, no condensation |
| Power Requirement | 12v ~ 16v DC, 0.9A ~ 0.7A |
| Power Consumption | Approximately 10.5W |
| Camera Mount | 1/4" - 20 UNC (top / bottom selectable) |
| Dimensions (approx) | 4.8(W) x 4.61(H) x 9.33(D) in. |
| Weight (approx) | 2,550g (89.9 oz.) |
| AC Adaptor |  |
| Power Requirement | 100 ~ 240V AC, 60 / 50 HZ |
| Input current | 1.3 A or less (Vin = AC 100v) |
| Dimensions | 1.95(W) x 1.06(H) x 4.51(D) in. |
| Weight (approx) | 270 g (9.5 oz) (without power cable) |

http://www.securitex.com.sg