## VCB-3524		Architect & Engineer Specification

### Bid Spec

- Description

The black and white video camera described shall be a Sanyo Model number VCB-3524 or equivalent. The Camera described shall be an industrial grade high sensitivity, high-resolution black and white video camera.

- Bid Specifications

The black and white video camera shall incorporate an interline transfer method 1/2” charge coupled device (CCD) with approx. 410,000 picture elements (811Hx508V, 768Hx494V effective). This camera shall produce a picture with more than 570 lines of resolution and have a minimum light requirement of 0.07 lux with F1.2 lens and a S/N ratio of more than 50dB. The video output level shall be 1.0Vp-p (75 ohms, composite) with a BNC type connection.

The camera shall be C-mount and CS-mount selectable with a C mount adapter (included). The camera shall include (1) mounting bracket (1/4”-20UNC) which can be mounted either on the top or the bottom of the camera. The video camera shall have an auto iris connector on the side of the camera body to permit connection of DC: iris control coil only type lenses (galvanometric iris without amplifier) and VIDEO: DC power and video signal supply auto-iris type lenses (with amplifier) type lenses.

The video camera shall have an available Backlight compensation circuit, Electronic/Auto Iris and Flange back adjustment. The camera shall have Line lock Sync. capability, automatic selection.

The camera shall not exceed a dimension of 2.2” x 1.77” x 3.92” (56x45x99.5mm) (w/o lens mount and camera mount) and weigh no more than 330 grams. Power requirements for the video camera shall be 24V AC  and consume 2.8W (approx. w/o Auto Iris lens). The video camera shall be constructed of a durable metal cabinet and shall be magnetic and electrostatically shielded. The camera shall feature solid state components to resist shock and vibration.  The video camera shall be UL listed.

- Minimum Performance Specifications

| Specifications |  |
| --- | --- |
| model | VCB-3524 |
| Scanning system | EIA Standard:525 lines, 30 frames/sec |
| Image device | Interline transfer method CCD |
| Image size | 1/2” (Approx. 6.4mm x 4.8mm) |
| Picture element | 811(H)x508(V):All picture elements 768(H)x494(V):Effective picture elements |
| Optical blank element | Horizontal 3:in front, 40:in back Vertical 12:in front, 2:in back |
| Unit (pixel) cell size | 8.4(H) x 9.8(V) m |
| Synchronizing system | Line Lock |
| Interlace | PLL2:1 interlace |
| Resolution | Horizontal 570 TV lines Vertical 350 TV lines |
| Video output level | 1.0Vp-p(75, composite) |
| Video S/N ratio | More than 50dB |
| Minimum Illumination | Scene: 0.07 lux(F1.2 lens), 0.09 lux(F1.4 lens) Imager: 0.01 lux |
| Gamma correction | 0.45 fixed (one possible internally) |
| Backlight compensation | ON/OFF, Manual switching, center zone light measuring system |
| Lens mount | CS mount (C mount: Using C-mount adapter supplied) |
| Flange back adjustment | 12.5mm –0.8 to +0.8mm |
| Camera mount | ¼” – 20 UNC x 2 (Top/Bottom) |
| Control | V Phase Adjust LINE PHASE VR Rear Lens Iris level LEVEL:L,H VR Side Auto iris lens A.I. lens:DC/VIDEO Slide SW Side Electronic Iris E.I.:ON/OFF SLIDE SW Side |
| Sockets | Video VIDEO OUT BNC Lens LENS OUT 4-pin Power Supply 24V AC, GND 3-pin terminal |
| Electronic Iris Range | 0.07 lux to 18,000 lux (F1.2 lens) 0.09 lux to 24,000 lux (F1.4 lens) 0.36 lux to 100,000 lux (F2.8 lens) |
| Light control | Electronic iris (indoor use) or optional auto iris lens |
| Auto iris output and connection | DC: iris control coil only type lens (Galvenometric iris without amplifier type Drive Coil (+-), Brake Coil (+-), Ground Socket Pin No.1 Brake Coil- Socket Pin No.2 Brake Coil+ Socket Pin No.3 Drive Coil+ Socket Pin No.4 Drive Coil- VIDEO:DC Power and Video signal supply auto-iris type lens (with amplifier type) +12V DC (MAX 50mA), Video(High impedance), Ground Socket Pin No.1 +12V DC (50mA max) Socket Pin No.2 Not Used Socket Pin No.3 Video Out(1Vp-p Hi Impedance) Socket Pin No.4 Drive Coil+ |
| Environmental Conditions | Operating: Temperature: 14F to 122F Humidity: Within 90% relative humidity Storage: Temperature -4F to 158F Humidity Within 70% relative humidity |
| Power Requirement | 24V AC (4V),60Hz |
| Size (WxHxD) | 2.2”x1.77”x3.92” (w/o lens mount and camera mount) |
| Weight (without lens) | .73lb (330g) |

http://www.securitex.com.sg