# VCC-4594		Architect & Engineer Specification___________________

## Bid Spec

- Description

The color video camera shall be a Sanyo Model number VCC-4594 or equivalent. The camera described shall be an industrial grade, high sensitivity, high-resolution, dual mode color and black and white video camera with digital signal processing.

- Bid Specification

The color camera shall incorporate an interline transfer method 1/3” charge coupled device (CCD) with approx. 410,000 picture elements (811H x 508V, 768H x 494V effective). The color video camera shall produce a picture with more than 520 lines of resolution in color and 520 lines of resolution in black and white. The video camera shall have the ability to switch from color mode to black and white mode automatically in low light conditions. The video camera shall have a minimum light requirement of 0.6 lx at F1.2 (color mode) or 0.03 lx at F1.2 (black & white mode). The video camera shall have a video signal to noise ratio of more than 52db. The video output level shall be 1.0 Vp-p (75 ohms composite) with a BNC type connection.

The video camera shall have advanced intelligence to eliminate light hunting and picture distortions caused by color/black and white switching. The video camera shall have a color/black and white level control switch. While in the black & white mode, the video camera shall have an infrared lighting response greater than 930 nanometers, thus allowing the camera to produce a picture where no visible light is available.

The camera shall be C-mount and CS-mount selectable with a C mount adapter (included). The camera shall include (1) mounting bracket (1/4”-20UNC) which can be mounted either on the top or the bottom of the camera. The video camera shall have an auto iris connector on the side of the camera body to permit connection of DC: iris control coil only type lenses (galvanometric iris without amplifier) and VIDEO: DC power and video signal supply auto-iris type lenses (with amplifier) type lenses. The video camera shall have a flange back adjustment.

The video camera shall have an available multi zone Backlight compensation circuit with multi-zone, white balance (auto and manual adjustment available), Electronic Iris/Auto Iris, Gamma correction (0.45), and flange back adjustment. The camera shall have internal, line lock capabilities.

The color video camera shall not exceed a dimension of 2.64” x 2.13” x 4.98” (67 x 54 x 126.5mm) and weigh no more than 15.9 0z. (450 grams). (w/o lens mount and camera mount) Power requirements for the video camera shall be 24V AC, 60 Hz and shall consume approx. 3.8W (with Auto Iris lens) and 2.9W (without Auto Iris lens). The video camera shall be constructed of a durable metal cabinet and shall be magnetically and electrostatically shielded. The camera shall feature solid state components to resist shock and vibration. The video camera shall be ISO9001 certified and UL listed.

3.0      Minimum Performance Specifications

| Specifications |  |
| --- | --- |
| MODEL | VCC-4594 |
| Scanning System | NTSC Standard 525 lines, 30 frames/Sec: 2:1 Interlace |
| Image Device | Interline Transfer CCD |
| Image Size | 1/3: (Appox. 4.8mm x 3.6mm) |
| Picture Element | 811 x 508 (V) picture elements |
|  | 768 (H) x 494 (V) effective picture elements |
| Unit (pixel) cell size | 6.35 (H) x 7.4 (V) um |
| Geometry | No Geometric Distortion |
| Synchronizing System | Internal/Line Lock Synchronization |
| Interlace | PLL 2:1 Interlace |
| Resolution | Color: Horizontal 520TV lines |
|  | Black & White: Horizontal 520TV lines |
| Video Output Level | 1.0 Vp-p @ 75 ohms composite |
| Video S/N Ratio | More than 52db |
| Minimum illumination | 0.03 lux @ F1.2 lens – black & white mode @ 50IRE |
| (incandescent lighting) | 0.6 lux @ F1.2 lens – Color @ 50IRE |
| Faceplate illumination | 0.094 ( F1.2 ) |
| Gamma correction | 0.45 |
| Gain Control (AGC) | Automatic |
| White Balance | ATW/Manual |
| Backlight Compensation | ON/OFF, Manual Switching, 2 Mode Setting: Center Zone Light |
|  | Measuring & Multi Zone Light Measuring |
| Lens Mount | CS Mount |
| Flange Back Adjustment | 12.5mm + 0.5mm |
| Camera Mount | 1/4" 20 UNC (top/bottom selectable) |
|  | Auto Iris Lens AI LENS:DC/VIDEO Slide Sw. (Side) |
|  | Electronic Iris ON/OFF Slide Sw. (Side) |
| Controls | Color Adjust Gain R. B Slide Sw (Side) |
|  | Backlight Compensation BLC: ON/OFF Slide Sw. (Slide) |
|  | Backlight Mode BLC: MULT/CENT Slide Sw. (Slide) |
|  | Line Lock Phase VR Control Slide Sw. (Slide) |
|  | White Balance ATW & Manual Slide Sw. (Slide) |
|  | Video VIDEO OUT BNC (Rear) |
| Sockets | Auto Iris Lens LENS OUT 4-pin (Side) |
|  | Power Supply 24V AC, GND 3-pin term. (Rear) |
| Electronic Iris Range | 0.6 lx to 35,000 lx @ F1.2 (color) 1.0 lx to 50,000 lx @ F1.4 (color) |
| Environmental Conditions | Operating: Temperature -10 degree C to +50 degree C |
|  | Humidity: Within 90% RH |
|  | Storage: Temperature -20 degree C to +70 degree C |
|  | Humidity: Within 70% RH |
| Power Indicator | LED |
| Power Requirement | 24V AC, 60Hz |
| Power Consumption | Approx. 3.8 W (with AI, lens) |
|  | Approx. 2.9 W (without AI, lens) |
| Dimensions (W x H x D) | 2.6 x 2.1 x 5 inches (67 x 54 x 128mm) |
|  | (Without lens mount and camera mount) |
| Weight | Approx. 450g (without lens) |

http://www.securitex.com.sg